# Practical 10: DNS and HTTP Configuration

Practical Title: Configure DNS mappings and simulate HTTP access.

Aim: To configure a DNS server to map a domain name to an IP address and to simulate web (HTTP) access from a client using Cisco Packet Tracer.

Objective:

- To understand the purpose of DNS.

- To configure a DNS server with an A record.

- To configure a web server with an HTML page.

- To verify HTTP access from a client using the domain name.

Theory: DNS (Domain Name System) is a hierarchical and decentralized naming system for computers, services, or other resources connected to the Internet or a private network. It translates domain names (e.g., www.example.com) into IP addresses (e.g.,

192.168.1.100), which computers use to locate each other. HTTP (Hypertext Transfer Protocol) is the protocol used to transfer data over the web.

Steps:

- Create the Network Topology:

  - Open Cisco Packet Tracer.

○ Create a simple topology with a PC, a switch, and a server. The server will act as both the DNS server and the web server.

- Assign IP Addresses:

  - PC: 192.168.1.10, Subnet Mask 255.255.255.0, Default Gateway 192.168.1.1, DNS Server: 192.168.1.20

○ Server: 192.168.1.20, Subnet Mask 255.255.255.0

○ You will need a router to act as the default gateway in a more complex setup, but for this simple practical, a switch connecting the PC and server is sufficient if they are in the same subnet.

- Configure the Web Server (HTTP Service):

  - Click on the Server > Services tab > HTTP.

○ Make sure the HTTP service is On. You can edit the index.html file to add a simple message, like "Welcome to the web server!".

- Configure the DNS Server (DNS Service):

  - Click on the Server > Services tab > DNS.

○ Make sure the DNS service is On.

○ Create a new A Record:

■ Name: www.example.com

■ Address: 192.168.1.20 ○ Click Add.

- Simulate HTTP Access:

  - On the PC, go to Desktop > Web Browser.

○ In the URL bar, type www.example.com and press Go.

○ The web page you created on the server should load. This demonstrates that the DNS server correctly translated the domain name to the IP address, allowing the HTTP request to succeed.

Conclusion: This practical successfully demonstrated the fundamental roles of DNS and HTTP. We configured a DNS server to map a human-readable domain name to an IP address and then used a web browser to access a web server using that domain name, showcasing the seamless interaction between these two essential protocols.

Viva / Oral Questions:

- What is the primary function of a DNS server?

- What is an A record in DNS?

- What is HTTP, and what is it used for?

- How does a web browser find a website's IP address when you type a URL?

- Why is a DNS server essential for the internet to work as we know it?